[image: image1.jpg]Hose Lift

“the 4th element”




Specification Data Sheet
Components:

· wire frame base

· aluminum tube, five section mast, with loop for cpap hose.
· plastic hook, mounted upon mast, for mask storage

· graphic instruction card, with list of optional accessories
· fabric travel bag, with drawstring closure

Wire Frame Base
· painted steel, size: 5” x 8” x 1”post”  (12.7 x 20.3 x 2.5 cm), wire diameter: 5/32” (4 mm), black; base inserts under, and is held by the weight of, any mattress system.
Aluminum Tubing Mast
· anodized blue aluminum tubing, 5 sections, each an approximate net 8” (20.3 cm) long and having a machined aluminum end fitting which inserts into the adjacent tube when erected; the 4th uppermost section - bent at 45 degrees to achieve reach; all sections held in order and erection sequence with an interior tensioned shock cord; tubing diameter: O.D. 3/8” (10 mm); I.D. 21/64” (8.2 mm).
· loop at uppermost end of mast:  formed from same shock cord within the tubing interior, cord approximately 1/8” (3 mm) in diameter; with a plastic “curly cord” applied over the cord loop to act as a roller for the cpap hose to move more freely, as it is held suspended above patient in bed.
· mast to base connection:  a plastic fitting affixed to lowest mast section, which sockets onto the bent wire frame base “post”; mast assembly shock cord end comes knotted, beneath base plastic fitting.

Instruction Card
· heavy card stock, 5” x 8” (12.7 x 20.3 cm), with 100% graphic display of instructions for use, on side one; side two displays optional accessories: painted steel fixed base for permanent bed frame mounting, and ground spike for camping use, all accessories are black.

Fabric Travel Bag

· approximately 7” x 11” (17.8 x 28 cm) 200 denier nylon; drawstring closure with spring actuated clasp; all black.
Overall Specifications

· erected height: approximately 37” (94 cm)

· collapsed travel size: approximately 5” x 8” x 1” (12.7 x 20.3 x 2.5 cm)
· total weight with all components including travel bag:  6 ¾ oz (19 grams).
Standard Packaging:

· Cardboard cartons, 16” x 11” x 9” (41 x 28 x 22 cm), 11 lbs 5 oz ( 5.1 kg).

· 25 CPAP Hose Lift “Units” per carton, said Units contained within poly bag, within each carton.

· each individual CPAP Hose Lift’s collapsed anodized aluminum mast, is wrapped in protective tissue paper, within it’s fabric travel bag. 
Manufacturer

· Arden Innovations, 1280 County Road 100, Georgetown, TX 78626, USA; CE mark #CA011750.
Manufacturer’s Suggested Retail Price per CPAP Hose Lift Unit for 2011:  $27  US Dollars each.
